Amucor Shield 6800屏蔽

| Amucor EMI shield is for panels and screwed applications该EMI屏蔽是用于面板和螺丝拧紧的应用 |  |
| --- | --- |
| Amucor Shield 6800 is an economical HF gasket which can be supplied in a wide range of dimensions. It is very effective in combination with zinc-plated steel and aluminium constructions. Amucor Shield 6800是一个经济的HF垫圈，能有一个宽范围尺寸的供货。它是非常有效的与锌板钢和铝结构结合。 All gaskets can be provided with a (conductive) self-adhesive strip. 所有的垫圈能被供应带一个(导电)自粘条。 Amucor Shield 6800 consists of a neoprene or PVC foam core covered with reinforced foil, based on an Amucor alloy. This guarantees excellent shielding performance and very great strength. Amucor Shield 6800由一个氯丁橡胶或PVC泡沫核覆盖有加固薄片组成，基于一个Amucor合金。这个确保卓越的屏蔽性能和非常大的强度。 For special applications, different foam cores and conductive foils and fabrics are available. 对于特殊的应用，不同的泡沫核和导电薄片和织物可获得。 | Flat panels and screwed applications. With and without self adhesive strip.平板和螺丝拧进的应用。带有和不带有自粘条。 |
| Shielding performance屏蔽性能 Shielding effectiveness depends on surface, shape of gaskets and material used. 屏蔽效率依据垫圈的表面、形状和使用的材料。 Frequency频率Hz; Mode; Screening dB屏蔽. 1M; E; 121. 10M; E; 110. 100M; E; 103. 400M; E; 98. 1G; P; 92. 10G; P; 89. See Guarantee. | Amucor gasket used on a Honeycomb ventilation panel Amucor垫圈使用在一个蜂窝通风面板上 |
| Benefits好处 - Self-adhesive EMC gasket 自粘EMC垫圈 - Easy to fit, can be cut with scissors 容易安装，能用剪刀剪切 - Good water resistance 良好的抗水性 - Gasket can be die-cut (screw holes) 垫圈能被冲切（螺丝孔） - Roll length up to 50, 100, 250 or 1000 m 卷的长度可达50,100,250或1000米。 - High shielding performance 高的屏蔽性能 - Low closure force 低的闭合力 - EXTREMELY STRONG; 非常牢固 - Deflection屈折 50% | Very small Amucor gasket, 1mm width and 1mm high versions are available 非常小的Amucor垫圈，1mm宽度和1mm高度的版本可获得 |
| Options选件 - Cut into accurate lengths 切成精确的长度 - Can be made die cut or as frame 能被做冲切或作为框架 - Combination with water seal 与水封结合 - UL94V-0 flame retardant foam core 阻燃剂泡沫核 - Chemical resistant rubbers like EPDM 耐化学橡胶，如EPDM - Silicone sponge for high temperatures up to 220°C; - 硅树脂海绵用于高温直到220°C; - Different conductive foils and fabrics - 不同的导电箔和织物 | Amucor shield can be delivered on rolls, worldwide delivery |
| Mechanical properties机械性能 | Standard gasket dimensions标准垫圈尺寸 |

Dimensions尺寸

| Width X(mm)宽度 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Height高度 Y(mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 10 | 12 | 15 | 18 | 20 | 25 | 32 | 50 |
|  | 1 | 6821 | 6831 | 6841 | 6851 | 6861 | 6871 | 6881 | 68101 | 68121 | 68151 | 68181 | 68201 | 68251 | 68321 | 68501 |
|  | 2 | 6822 | 6832 | 6842 | 6852 | 6862 | 6872 | 6882 | 68102 | 68122 | 68152 | 68182 | 68202 | 68252 | 68322 | 68502 |
|  | 3 |  | 6833 | 6843 | 6853 | 6863 | 6873 | 6883 | 68103 | 68123 | 68153 | 68183 | 68203 | 68253 | 68323 | 68503 |
|  | 4 |  |  | 6844 | 6854 | 6864 | 6874 | 6884 | 68104 | 68124 | 68154 | 68184 | 68204 | 68254 | 68324 | 68504 |
|  | 5 |  |  |  | 6855 | 6865 | 6875 | 6885 | 68105 | 68125 | 68155 | 68185 | 68205 | 68255 | 68325 | 68505 |
|  | 6 |  |  |  |  | 6866 | 6876 | 6886 | 68106 | 68126 | 68156 | 68186 | 68206 | 68256 | 68326 | 68506 |
|  | 8 |  |  |  |  |  | 6878 | 6888 | 68108 | 68128 | 68158 | 68188 | 68208 | 68258 | 68328 | 68508 |
|  | 10 |  |  |  |  |  |  |  | 681010 | 681210 | 681510 | 681810 | 682010 | 682510 | 683210 | 685010 |
|  | 12 |  |  |  |  |  |  |  |  | 681212 | 681512 | 681812 | 682012 | 682512 | 683212 | 685012 |

Item marked bold are 98% stock items. Other dimensions on request.

项目记号黑粗体的是98%存货项目。其它的尺寸根据需求。

Tape Specification粘合带规格说明

- 01 : standard self-adhesive placed in the middle 标准自粘放置在中部

- 02 : without self-adhesive 不带有自粘

- 03 : with conductive self-adhesive 带有导电自粘

- 06 : standard self-adhesive, asymmetrical 标准自粘，不对称的

- 07 : standard self-adhesive placed on the side 标准自粘放置在侧面上

Foam Specification泡沫规格

- N : Standard Neoprene Foam 标准氯丁橡胶泡沫

- E : EPDM foam core 三元乙丙橡胶泡沫核

- P : low closure force PVC foam, slow recovery 低闭合力PVC泡沫，慢复原

- F : flame retardant UL94V-0 foam 阻燃剂UL94V-0泡沫

Example举例

When you order Amucor shield 6800, 8 x 2 mm, standard self-adhesive placed in the middle, standard neoprene foam core: 6882-01-N

当你订购Amucor shield 6800, 8 x 2 mm,标准自粘放置在中部，标准氯丁橡胶泡沫核：6882-01-N

| Part number零件号 | Tape code带编码 | Foam code泡沫编码 |
| --- | --- | --- |
| 6882 | 01 | N |
|  | 01 : standard self-adhesive placed in the middle 标准自粘放置在中部 02 : without self-adhesive 不带有自粘 03 : with conductive self-adhesive (only recommended on small sizes) 带有导电自粘（仅被推荐在小尺寸上） 06 : standard self-adhesive, asymmetrical标准自粘，不对称的 07 : standard self-adhesive placed on the side标准自粘放置在侧面 | N : standard neoprene foam标准氯丁橡胶泡沫 E : EPDM foam core三元乙丙橡胶核 P : PVC foam, slow recovery聚氯乙烯泡沫，慢复原 F : flame retardant UL94V-0 foam阻燃剂UL94V-0泡沫 |