DIGITAL PANEL METERS VNE 20

Electrical measurements 20000 points

VNE 20 panel meter are characterised by their high accuracy and resolution.

They permit the measurement hold by external command ant have an analog output.

The dot position can be selected by external command.

- Dimensions : 96 x 24 mm

- Built-in line power supply

- Display : 14 mm red, green or yellow LED

Functions

(1) En ± (% of reading + number of units) at 23 ± 1°C for 1 year.

General characteristics

Display……………………………..

20000 measurement points (4 ½ digits), Red, green or yellow LED, height : 14 mm, with brightness adjustment.

Automatic polarity and zeroing.

Measurement rate………………..

2.5 measurements per second

Measurement hold and dot commutation……………………...

By external command

Temperature coefficient………...

≤ (0.01 % + 0.2) / °C

Analog output…………………….

On all models

0.1 mV / digit, non isolated output.

Common mode acceptable…….

250 V - or ~.

Overload acceptable…………….

250 V – or ~.

Operating conditions……………

Nominal field of use : 0 to + 50 °C

Limit field of use : -10 to + 50°C

Rejection rate at 50 Hz………….

Serial mode : 80 dB

Common mode : 140 dB

Operating conditions……………

Nominal field of use : 0 to +50°C.

RH : 20 to 80 % without condensation.

Limit field of use : -10 to +50°C.

Power supply………………………………

230 V~ (± 10 %) 50 to 60 Hz

Other power supplies :

6 V- ± 20 % non insulated,

5 V- ± 10 % insulated,

9 to 18 V- insulated,

18 to 36 V- insulated,

36 to 72 V- insulated,

24 V~ ± 10 % insulated,

48 V~ ± 10 % insulated,

115 V~ ± 10 % insulated.

Other on request.

Consumption………………………………

4VA max.

Presentation……………………………….

ABS case.

IP40 Impervious.

Weight : 300 g.

Connection…………………………………

Removable screwing connector

Dimensions

Dimensions unit : mm

Instructions to order