SAS-521F-7 Bi-Logical Antenna Data

1 Meter | 3 Meter | 10 Meter
Antenna Page | VSWR Graph | AF Graph

- Typical Antenna Factor Plot

- Typical VSWR

- Mounting Diagram for the bilogical antenna series

SAS-521F-7 Bi-Logical Antenna Data

1 Meter | 3 Meter | 10 Meter
Antenna Page | VSWR Graph | AF Graph

| 1 Meter Antenna Factor |  |  |  |
| --- | --- | --- | --- |
| Frequency (MHz) | AF (dB/m) | Gain (dBi) | Gain (Numeric) |
| 25 | 26.0 | -27.8 | 0.00 |
| 30 | 23.5 | -23.7 | 0.00 |
| 35 | 21.5 | -20.4 | 0.01 |
| 40 | 16.8 | -14.5 | 0.04 |
| 45 | 14.4 | -11.1 | 0.08 |
| 50 | 9.5 | -5.3 | 0.29 |
| 60 | 11.3 | -5.5 | 0.28 |
| 70 | 12.4 | -5.3 | 0.30 |
| 80 | 15.1 | -6.7 | 0.21 |
| 90 | 19.3 | -10.0 | 0.10 |
| 100 | 20.8 | -10.6 | 0.09 |
| 125 | 23.4 | -11.2 | 0.08 |
| 150 | 25.5 | -11.8 | 0.07 |
| 175 | 20.3 | -5.2 | 0.30 |
| 200 | 15.5 | 0.8 | 1.21 |
| 225 | 15.2 | 2.1 | 1.62 |
| 250 | 16.9 | 1.3 | 1.34 |
| 275 | 16.2 | 2.8 | 1.91 |
| 300 | 15.6 | 4.2 | 2.61 |
| 325 | 16.7 | 3.8 | 2.41 |
| 350 | 16.2 | 4.9 | 3.11 |
| 375 | 17.2 | 4.5 | 2.84 |
| 400 | 17.9 | 4.3 | 2.72 |
| 425 | 18.2 | 4.6 | 2.87 |
| 450 | 18.1 | 5.2 | 3.29 |
| 475 | 19.1 | 4.6 | 2.91 |
| 500 | 18.9 | 5.3 | 3.38 |
| 525 | 19.0 | 5.6 | 3.66 |
| 550 | 21.2 | 3.9 | 2.45 |
| 575 | 20.7 | 4.7 | 2.96 |
| 600 | 20.4 | 5.4 | 3.44 |
| 625 | 20.6 | 5.6 | 3.63 |
| 650 | 21.7 | 4.8 | 3.02 |
| 675 | 21.5 | 5.4 | 3.45 |
| 700 | 21.9 | 5.3 | 3.38 |
| 725 | 22.5 | 4.9 | 3.11 |
| 750 | 22.6 | 5.2 | 3.30 |
| 775 | 23.0 | 5.0 | 3.17 |
| 800 | 22.9 | 5.4 | 3.45 |
| 825 | 23.4 | 5.1 | 3.27 |
| 850 | 24.1 | 4.7 | 2.95 |
| 875 | 23.9 | 5.2 | 3.30 |
| 900 | 24.6 | 4.8 | 2.99 |
| 925 | 24.5 | 5.0 | 3.18 |
| 950 | 25.4 | 4.4 | 2.74 |
| 975 | 25.5 | 4.5 | 2.81 |
| 1000 | 26.1 | 4.2 | 2.62 |
| 1100 | 27.3 | 3.7 | 2.37 |
| 1200 | 27.6 | 4.2 | 2.62 |
| 1300 | 28.1 | 4.4 | 2.77 |
| 1400 | 28.5 | 4.7 | 2.94 |
| 1500 | 28.9 | 4.8 | 3.04 |
| 1600 | 29.5 | 4.8 | 3.05 |
| 1700 | 29.6 | 5.2 | 3.34 |
| 1800 | 30.4 | 5.0 | 3.14 |
| 1900 | 31.2 | 4.6 | 2.92 |
| 2000 | 31.7 | 4.6 | 2.89 |
| 2100 | 32.3 | 4.3 | 2.72 |
| 2200 | 32.5 | 4.6 | 2.88 |
| 2300 | 32.9 | 4.6 | 2.87 |
| 2400 | 33.4 | 4.5 | 2.79 |
| 2500 | 33.7 | 4.5 | 2.81 |
| 2600 | 33.8 | 4.7 | 2.96 |
| 2700 | 33.9 | 4.9 | 3.12 |
| 2800 | 34.9 | 4.3 | 2.70 |
| 2900 | 35.5 | 4.0 | 2.51 |
| 3000 | 35.3 | 4.4 | 2.78 |
| 3100 | 35.7 | 4.3 | 2.72 |
| 3200 | 36.3 | 4.1 | 2.54 |
| 3300 | 37.1 | 3.5 | 2.24 |
| 3400 | 37.8 | 3.1 | 2.03 |
| 3500 | 37.5 | 3.6 | 2.29 |
| 3600 | 37.1 | 4.3 | 2.67 |
| 3700 | 37.6 | 4.0 | 2.50 |
| 3800 | 38.2 | 3.6 | 2.32 |
| 3900 | 38.3 | 3.8 | 2.37 |
| 4000 | 38.9 | 3.4 | 2.20 |
| 4100 | 39.6 | 2.9 | 1.97 |
| 4200 | 39.4 | 3.3 | 2.12 |
| 4300 | 39.2 | 3.7 | 2.37 |
| 4400 | 39.6 | 3.6 | 2.27 |
| 4500 | 39.9 | 3.4 | 2.19 |
| 4600 | 39.9 | 3.6 | 2.30 |
| 4700 | 40.2 | 3.5 | 2.25 |
| 4800 | 40.8 | 3.1 | 2.02 |
| 4900 | 40.9 | 3.2 | 2.08 |
| 5000 | 41.1 | 3.1 | 2.04 |
| 5100 | 41.2 | 3.2 | 2.09 |
| 5200 | 41.2 | 3.4 | 2.19 |
| 5300 | 41.4 | 3.3 | 2.15 |
| 5400 | 41.4 | 3.5 | 2.26 |
| 5500 | 41.8 | 3.3 | 2.12 |
| 5600 | 42.3 | 2.9 | 1.95 |
| 5700 | 43.1 | 2.3 | 1.70 |
| 5800 | 43.3 | 2.2 | 1.68 |
| 5900 | 44.3 | 1.3 | 1.36 |
| 6000 | 45.3 | 0.5 | 1.13 |
| 6100 | 44.8 | 1.2 | 1.31 |
| 6200 | 45.4 | 0.7 | 1.19 |
| 6300 | 45.5 | 0.7 | 1.18 |
| 6400 | 45.4 | 1.0 | 1.25 |
| 6500 | 45.8 | 0.7 | 1.18 |
| 6600 | 46.3 | 0.3 | 1.07 |
| 6700 | 47.3 | -0.5 | 0.89 |
| 6800 | 47.6 | -0.7 | 0.86 |
| 6900 | 48.5 | -1.4 | 0.72 |
| 7000 | 49.8 | -2.7 | 0.54 |

| 3 Meter Antenna Factor |  |  |  |
| --- | --- | --- | --- |
| Frequency (MHz) | AF (dB/m) | Gain (dBi) | Gain (Numeric) |
| 25 | 21.1 | -22.9 | 0.01 |
| 30 | 18.4 | -18.6 | 0.01 |
| 35 | 18.4 | -17.3 | 0.02 |
| 40 | 13.2 | -10.9 | 0.08 |
| 45 | 13.2 | -9.9 | 0.10 |
| 50 | 9.1 | -4.9 | 0.32 |
| 60 | 7.0 | -1.2 | 0.75 |
| 70 | 6.8 | 0.3 | 1.07 |
| 80 | 8.5 | -0.2 | 0.95 |
| 90 | 11.6 | -2.3 | 0.59 |
| 100 | 14.3 | -4.0 | 0.39 |
| 125 | 17.4 | -5.2 | 0.30 |
| 150 | 17.6 | -3.9 | 0.41 |
| 175 | 17.1 | -1.9 | 0.64 |
| 200 | 12.5 | 3.8 | 2.39 |
| 225 | 12.2 | 5.1 | 3.22 |
| 250 | 14.8 | 3.4 | 2.19 |
| 275 | 14.1 | 5.0 | 3.14 |
| 300 | 14.1 | 5.7 | 3.72 |
| 325 | 15.0 | 5.5 | 3.58 |
| 350 | 14.8 | 6.3 | 4.29 |
| 375 | 16.0 | 5.7 | 3.72 |
| 400 | 16.7 | 5.5 | 3.59 |
| 425 | 17.5 | 5.3 | 3.39 |
| 450 | 17.6 | 5.8 | 3.76 |
| 475 | 18.2 | 5.6 | 3.61 |
| 500 | 18.4 | 5.8 | 3.81 |
| 525 | 19.6 | 5.1 | 3.22 |
| 550 | 20.1 | 5.0 | 3.13 |
| 575 | 20.3 | 5.1 | 3.25 |
| 600 | 19.9 | 5.9 | 3.86 |
| 625 | 20.3 | 5.8 | 3.83 |
| 650 | 21.1 | 5.4 | 3.50 |
| 675 | 21.2 | 5.6 | 3.65 |
| 700 | 21.4 | 5.8 | 3.78 |
| 725 | 21.9 | 5.6 | 3.63 |
| 750 | 22.2 | 5.5 | 3.58 |
| 775 | 22.4 | 5.6 | 3.66 |
| 800 | 22.6 | 5.7 | 3.73 |
| 825 | 23.1 | 5.5 | 3.57 |
| 850 | 23.1 | 5.7 | 3.73 |
| 875 | 23.3 | 5.8 | 3.78 |
| 900 | 23.5 | 5.9 | 3.85 |
| 925 | 23.8 | 5.8 | 3.77 |
| 950 | 24.1 | 5.7 | 3.73 |
| 975 | 24.1 | 5.9 | 3.91 |
| 1000 | 24.4 | 5.8 | 3.84 |
| 1100 | 24.9 | 6.2 | 4.14 |
| 1200 | 25.6 | 6.2 | 4.18 |
| 1300 | 26.4 | 6.1 | 4.08 |
| 1400 | 27.4 | 5.8 | 3.77 |
| 1500 | 28.1 | 5.7 | 3.72 |
| 1600 | 29.0 | 5.4 | 3.45 |
| 1700 | 29.3 | 5.6 | 3.62 |
| 1800 | 29.8 | 5.6 | 3.60 |
| 1900 | 30.5 | 5.3 | 3.41 |
| 2000 | 30.9 | 5.3 | 3.41 |
| 2100 | 31.8 | 4.9 | 3.09 |
| 2200 | 31.4 | 5.7 | 3.67 |
| 2300 | 31.7 | 5.7 | 3.75 |
| 2400 | 32.5 | 5.4 | 3.43 |
| 2500 | 32.7 | 5.5 | 3.53 |
| 2600 | 32.4 | 6.1 | 4.07 |
| 2700 | 32.7 | 6.2 | 4.15 |
| 2800 | 33.7 | 5.5 | 3.55 |
| 2900 | 34.1 | 5.4 | 3.44 |
| 3000 | 33.9 | 5.9 | 3.89 |
| 3100 | 34.0 | 6.0 | 4.02 |
| 3200 | 34.6 | 5.8 | 3.80 |
| 3300 | 35.1 | 5.5 | 3.58 |
| 3400 | 36.1 | 4.8 | 3.04 |
| 3500 | 35.6 | 5.5 | 3.59 |
| 3600 | 35.1 | 6.3 | 4.23 |
| 3700 | 35.7 | 5.9 | 3.91 |
| 3800 | 36.9 | 5.0 | 3.14 |
| 3900 | 37.3 | 4.7 | 2.98 |
| 4000 | 37.5 | 4.8 | 3.01 |
| 4100 | 38.1 | 4.4 | 2.74 |
| 4200 | 38.3 | 4.4 | 2.77 |
| 4300 | 38.4 | 4.6 | 2.86 |
| 4400 | 38.3 | 4.8 | 3.01 |
| 4500 | 38.8 | 4.6 | 2.85 |
| 4600 | 38.9 | 4.6 | 2.89 |
| 4700 | 39.2 | 4.5 | 2.83 |
| 4800 | 39.4 | 4.5 | 2.79 |
| 4900 | 39.9 | 4.2 | 2.61 |
| 5000 | 40.0 | 4.3 | 2.68 |
| 5100 | 39.8 | 4.6 | 2.85 |
| 5200 | 39.5 | 5.1 | 3.24 |
| 5300 | 40.1 | 4.7 | 2.93 |
| 5400 | 39.6 | 5.3 | 3.36 |
| 5500 | 40.3 | 4.8 | 2.99 |
| 5600 | 40.5 | 4.7 | 2.95 |
| 5700 | 41.1 | 4.3 | 2.69 |
| 5800 | 41.8 | 3.7 | 2.34 |
| 5900 | 42.4 | 3.2 | 2.10 |
| 6000 | 43.3 | 2.5 | 1.77 |
| 6100 | 43.6 | 2.4 | 1.72 |
| 6200 | 43.4 | 2.7 | 1.85 |
| 6300 | 44.2 | 2.0 | 1.58 |
| 6400 | 44.5 | 1.9 | 1.55 |
| 6500 | 44.6 | 1.9 | 1.56 |
| 6600 | 45.0 | 1.7 | 1.47 |
| 6700 | 45.8 | 1.0 | 1.26 |
| 6800 | 46.6 | 0.3 | 1.08 |
| 6900 | 47.0 | 0.0 | 0.99 |
| 7000 | 48.5 | -1.3 | 0.73 |

| 10 Meter Antenna Factor |  |  |  |
| --- | --- | --- | --- |
| Frequency (MHz) | AF (dB/m) | Gain (dBi) | Gain (Numeric) |
| 25 | 19.2 | -21.0 | 0.01 |
| 30 | 17.2 | -17.4 | 0.02 |
| 35 | 17.2 | -16.0 | 0.02 |
| 40 | 12.4 | -10.1 | 0.10 |
| 45 | 12.4 | -9.1 | 0.12 |
| 50 | 8.9 | -4.6 | 0.34 |
| 60 | 6.4 | -0.6 | 0.88 |
| 70 | 6.6 | 0.5 | 1.12 |
| 80 | 8.5 | -0.2 | 0.96 |
| 90 | 12.1 | -2.7 | 0.53 |
| 100 | 13.0 | -2.8 | 0.53 |
| 125 | 17.3 | -5.2 | 0.31 |
| 150 | 16.2 | -2.5 | 0.57 |
| 175 | 15.9 | -0.8 | 0.83 |
| 200 | 11.1 | 5.2 | 3.31 |
| 225 | 11.3 | 6.0 | 3.98 |
| 250 | 14.2 | 4.0 | 2.49 |
| 275 | 13.2 | 5.8 | 3.80 |
| 300 | 13.4 | 6.4 | 4.34 |
| 325 | 14.3 | 6.2 | 4.17 |
| 350 | 14.0 | 7.1 | 5.17 |
| 375 | 15.6 | 6.1 | 4.09 |
| 400 | 16.4 | 5.9 | 3.91 |
| 425 | 17.1 | 5.7 | 3.71 |
| 450 | 16.9 | 6.4 | 4.34 |
| 475 | 17.8 | 6.0 | 4.01 |
| 500 | 18.2 | 6.0 | 4.03 |
| 525 | 19.3 | 5.4 | 3.46 |
| 550 | 19.4 | 5.7 | 3.68 |
| 575 | 19.8 | 5.7 | 3.70 |
| 600 | 19.4 | 6.4 | 4.33 |
| 625 | 19.8 | 6.3 | 4.30 |
| 650 | 20.6 | 5.9 | 3.93 |
| 675 | 20.8 | 6.0 | 4.00 |
| 700 | 20.9 | 6.3 | 4.25 |
| 725 | 21.5 | 6.0 | 3.96 |
| 750 | 21.8 | 6.0 | 3.96 |
| 775 | 22.0 | 6.0 | 4.00 |
| 800 | 22.1 | 6.2 | 4.13 |
| 825 | 22.5 | 6.1 | 4.06 |
| 850 | 22.7 | 6.2 | 4.13 |
| 875 | 22.9 | 6.2 | 4.15 |
| 900 | 23.1 | 6.2 | 4.19 |
| 925 | 23.4 | 6.2 | 4.13 |
| 950 | 23.5 | 6.3 | 4.25 |
| 975 | 23.9 | 6.1 | 4.06 |
| 1000 | 23.9 | 6.3 | 4.27 |
| 1100 | 25.5 | 5.6 | 3.62 |
| 1200 | 26.0 | 5.8 | 3.82 |
| 1300 | 26.9 | 5.6 | 3.66 |
| 1400 | 27.6 | 5.5 | 3.58 |
| 1500 | 28.1 | 5.6 | 3.66 |
| 1600 | 29.4 | 4.9 | 3.11 |
| 1700 | 29.2 | 5.7 | 3.69 |
| 1800 | 29.8 | 5.6 | 3.61 |
| 1900 | 30.3 | 5.5 | 3.54 |
| 2000 | 30.8 | 5.5 | 3.55 |
| 2100 | 31.1 | 5.6 | 3.59 |
| 2200 | 30.0 | 7.1 | 5.16 |
| 2300 | 30.1 | 7.3 | 5.42 |
| 2400 | 31.6 | 6.3 | 4.26 |
| 2500 | 31.4 | 6.8 | 4.81 |
| 2600 | 31.0 | 7.5 | 5.67 |
| 2700 | 31.6 | 7.2 | 5.29 |
| 2800 | 32.0 | 7.2 | 5.26 |
| 2900 | 32.6 | 6.8 | 4.84 |
| 3000 | 32.5 | 7.3 | 5.39 |
| 3100 | 32.0 | 8.1 | 6.47 |
| 3200 | 33.0 | 7.4 | 5.48 |
| 3300 | 33.1 | 7.5 | 5.63 |
| 3400 | 34.3 | 6.5 | 4.50 |
| 3500 | 33.9 | 7.2 | 5.29 |
| 3600 | 33.0 | 8.3 | 6.82 |
| 3700 | 34.5 | 7.2 | 5.20 |
| 3800 | 35.6 | 6.2 | 4.21 |
| 3900 | 36.4 | 5.6 | 3.66 |
| 4000 | 36.1 | 6.2 | 4.16 |
| 4100 | 37.2 | 5.3 | 3.42 |
| 4200 | 37.0 | 5.7 | 3.72 |
| 4300 | 37.6 | 5.4 | 3.44 |
| 4400 | 37.1 | 6.0 | 3.98 |
| 4500 | 36.8 | 6.5 | 4.50 |
| 4600 | 38.0 | 5.5 | 3.57 |
| 4700 | 38.0 | 5.7 | 3.71 |
| 4800 | 37.8 | 6.0 | 4.03 |
| 4900 | 38.3 | 5.8 | 3.76 |
| 5000 | 38.4 | 5.9 | 3.86 |
| 5100 | 38.8 | 5.6 | 3.62 |
| 5200 | 37.8 | 6.8 | 4.77 |
| 5300 | 38.6 | 6.1 | 4.12 |
| 5400 | 37.8 | 7.1 | 5.08 |
| 5500 | 38.6 | 6.4 | 4.38 |
| 5600 | 37.9 | 7.3 | 5.41 |
| 5700 | 39.6 | 5.8 | 3.80 |
| 5800 | 39.7 | 5.8 | 3.78 |
| 5900 | 40.0 | 5.7 | 3.68 |
| 6000 | 42.1 | 3.7 | 2.36 |
| 6100 | 42.3 | 3.7 | 2.34 |
| 6200 | 41.8 | 4.3 | 2.70 |
| 6300 | 43.4 | 2.8 | 1.91 |
| 6400 | 42.1 | 4.2 | 2.65 |
| 6500 | 44.1 | 2.4 | 1.72 |
| 6600 | 42.7 | 3.9 | 2.45 |
| 6700 | 43.9 | 2.8 | 1.93 |
| 6800 | 45.0 | 1.9 | 1.56 |
| 6900 | 45.5 | 1.6 | 1.43 |
| 7000 | 47.1 | 0.0 | 1.01 |