FCC-2 Tuned Dipole Antenna Data

More Information

- Typical Antenna Factor Plot - 3m

Innovation, Quality, and Performance

- Typical Antenna Factor Plot - 10m

Innovation, Quality, and Performance

- Typical VSWR

Innovation, Quality, and Performance

- Numeric Data

FCC-2 Tuned Dipole Antenna Data

3 Meter | 10 Meter
Antenna Page | VSWR Graph | AF Graph

| 3 Meter Antenna Factor |  |  |  |
| --- | --- | --- | --- |
| Frequency (MHz) | Element Length (L/2-in.) | Antenna Factor (dB/m) | Gain (dBi) |
| 65 | 46 1/4 | 4.9 | 1.6 |
| 70 | 43 | 5.4 | 1.7 |
| 75 | 39 7/8 | 5.9 | 1.8 |
| 80 | 36 3/4 | 6.5 | 1.8 |
| 85 | 33 3/4 | 7.1 | 1.7 |
| 90 | 32 5/8 | 7.5 | 1.8 |
| 95 | 30 5/8 | 7.9 | 1.9 |
| 100 | 28 3/4 | 8.4 | 1.8 |
| 105 | 27 1/4 | 8.9 | 1.8 |
| 110 | 25 7/8 | 9.3 | 1.8 |
| 115 | 24 5/8 | 9.7 | 1.8 |
| 120 | 23 3/8 | 10.1 | 1.7 |
| 125 | 22 3/8 | 10.8 | 1.4 |
| 130 | 21 1/2 | 11.3 | 1.2 |
| 135 | 20 3/4 | 11.7 | 1.2 |
| 140 | 20 1/8 | 11.8 | 1.4 |
| 145 | 19 3/8 | 12.1 | 1.4 |
| 150 | 18 3/4 | 11.9 | 1.9 |
| 155 | 18 | 12.1 | 2.0 |
| 160 | 17 1/4 | 12.4 | 1.9 |
| 165 | 16 5/8 | 12.7 | 1.9 |
| 170 | 16 1/8 | 12.8 | 2.1 |
| 175 | 15 5/8 | 13.3 | 1.8 |
| 180 | 15 3/16 | 13.8 | 1.6 |

| 10 Meter Antenna Factor |  |  |  |
| --- | --- | --- | --- |
| Frequency (MHz) | Element Length (L/2-in.) | Antenna Factor (dB/m) | Gain (dBi) |
| 65 | 46 1/4 | 4.7 | 1.8 |
| 70 | 43 | 5.2 | 1.9 |
| 75 | 39 7/8 | 5.8 | 1.9 |
| 80 | 36 3/4 | 6.4 | 1.9 |
| 85 | 33 3/4 | 6.8 | 2.0 |
| 90 | 32 5/8 | 7.4 | 1.9 |
| 95 | 30 5/8 | 7.8 | 2.0 |
| 100 | 28 3/4 | 8.3 | 1.9 |
| 105 | 27 1/4 | 8.9 | 1.8 |
| 110 | 25 7/8 | 9.2 | 1.9 |
| 115 | 24 5/8 | 9.5 | 2.0 |
| 120 | 23 3/8 | 10.2 | 1.6 |
| 125 | 22 3/8 | 10.4 | 1.8 |
| 130 | 21 1/2 | 10.9 | 1.6 |
| 135 | 20 3/4 | 11.4 | 1.5 |
| 140 | 20 1/8 | 11.3 | 1.9 |
| 145 | 19 3/8 | 11.5 | 2.0 |
| 150 | 18 3/4 | 11.8 | 2.0 |
| 155 | 18 | 12.1 | 2.0 |
| 160 | 17 1/4 | 12.3 | 2.0 |
| 165 | 16 5/8 | 12.8 | 1.8 |
| 170 | 16 1/8 | 13 | 1.9 |
| 175 | 15 5/8 | 13.2 | 1.9 |
| 180 | 15 3/16 | 13.7 | 1.7 |