FCC-4 Tuned Dipole Antenna Data

More Information

- Typical Antenna Factor Plot - 3m

Innovation, Quality, and Performance

- Typical Antenna Factor Plot - 10m

Innovation, Quality, and Performance

- Typical VSWR

Innovation, Quality, and Performance

- Numeric Data

FCC-4 Tuned Dipole Antenna Data

3 Meter | 10 Meter
Antenna Page | VSWR Graph | AF Graph

| 3 Meter Antenna Factor |  |  |  |
| --- | --- | --- | --- |
| Frequency (MHz) | Element Length (L/2-in.) | Antenna Factor (dB/m) | Gain (dBi) |
| 325 | 8 1/8 | 19.4 | 1.1 |
| 350 | 7 3/4 | 20.2 | 0.9 |
| 375 | 7 1/2 | 21 | 0.7 |
| 400 | 7 1/8 | 21.4 | 0.9 |
| 425 | 6 9/16 | 22.2 | 0.6 |
| 450 | 6 1/8 | 22.2 | 1.1 |
| 475 | 5 5/8 | 22.9 | 0.9 |
| 500 | 5 1/4 | 23.2 | 1.0 |
| 525 | 4 15/16 | 23.4 | 1.2 |
| 550 | 4 3/4 | 23.8 | 1.3 |
| 575 | 4 1/2 | 24.2 | 1.2 |
| 600 | 4 3/8 | 24.7 | 1.1 |
| 625 | 4 1/8 | 25 | 1.2 |
| 650 | 4 | 25.2 | 1.3 |
| 675 | 3 13/16 | 25.6 | 1.2 |
| 700 | 3 11/16 | 25.8 | 1.3 |
| 725 | 3 7/16 | 26.2 | 1.3 |
| 750 | 3 1/4 | 26.5 | 1.2 |
| 775 | 3 1/8 | 26.9 | 1.1 |
| 800 | 3 | 27.1 | 1.2 |
| 825 | 2 15/16 | 27 | 1.6 |
| 850 | 2 7/8 | 27.9 | 0.9 |
| 875 | 2 11/16 | 27.8 | 1.3 |
| 900 | 2 5/8 | 28 | 1.3 |
| 925 | 2 5/8 | 28.5 | 1.1 |
| 950 | 2 5/8 | 28.8 | 1.0 |
| 975 | 2 5/8 | 29.1 | 0.9 |
| 1000 | 2 5/8 | 29 | 1.2 |

| 10 Meter |  |  |  |
| --- | --- | --- | --- |
| Frequency (MHz) | Element Length (L/2-in.) | Antenna Factor (dB/m) | Gain (dBi) |
| 325 | 8 1/8 | 19.8 | 0.7 |
| 350 | 7 3/4 | 20.5 | 0.6 |
| 375 | 7 1/2 | 20.8 | 0.9 |
| 400 | 7 1/8 | 21.1 | 1.2 |
| 425 | 6 9/16 | 21.4 | 1.4 |
| 450 | 6 1/8 | 21.5 | 1.8 |
| 475 | 5 5/8 | 22.1 | 1.7 |
| 500 | 5 1/4 | 22.9 | 1.3 |
| 525 | 4 15/16 | 23.8 | 0.8 |
| 550 | 4 3/4 | 24.1 | 1.0 |
| 575 | 4 1/2 | 24.7 | 0.7 |
| 600 | 4 3/8 | 25.1 | 0.7 |
| 625 | 4 1/8 | 25.6 | 0.6 |
| 650 | 4 | 25.5 | 1.0 |
| 675 | 3 13/16 | 25.7 | 1.1 |
| 700 | 3 11/16 | 25.7 | 1.4 |
| 725 | 3 7/16 | 26.1 | 1.4 |
| 750 | 3 1/4 | 26.2 | 1.5 |
| 775 | 3 1/8 | 26 | 2.0 |
| 800 | 3 | 26.3 | 2.0 |
| 825 | 2 15/16 | 26.6 | 2.0 |
| 850 | 2 7/8 | 26.8 | 2.0 |
| 875 | 2 11/16 | 27.2 | 1.9 |
| 900 | 2 5/8 | 27.6 | 1.7 |
| 925 | 2 5/8 | 27.9 | 1.7 |
| 950 | 2 5/8 | 28.7 | 1.1 |
| 975 | 2 5/8 | 29.4 | 0.6 |
| 1000 | 2 5/8 | 30 | 0.2 |